## Test Procedure

### Power Test Procedure

- Check the connector natural pins voltage and compare it to predefined values (e.g., DI = 3.5V, DO = 0V, Power = 24V).

- If the pin voltage changes in response to simulation commands (such as “ch 1+”), verify that the system correctly adjusts the analog input (e.g., to 10V) and matches the expected data (e.g., 5V).

### Pullup Test Procedure

- Confirm IO box wiring for Ground (e.g., TB2.2, CH.1-) and Float (e.g., DO_1) pins according to HSID definitions.

- Precondition for the test is the pin  Power Test results are “Pass” and natural voltage is ~0 before proceeding.

- Connect a Pullup signal from the Controllino board and remeasure the pin voltage Ground pin should measure ~0V; Float pin should measure ~21V, dropping to ~0V when the relevant DO is closed.

### Logic Test Procedure

- Verify IO box wiring for digital input connectivity (e.g., DI_1) per HSID specifications.

- Establish a connection between two pins and confirm the digital input changes as defined by HSID.

- Preconditions:

  - Tested pin Power Test result = “Pass” and voltage below 4V

  - Connected pin Power Test result = “Pass” and voltage below 4V for digital input, or 10V for analog input.

## Failure Criteria:

- Any measurement outside the predefined tolerance (±1.5V).

- Spider data readings that deviate from the plan (e.g., DI = 1 instead of 0

- System failure to command analog or digital outputs (UDP/Spider issue).

- System failure to communicate with the Controllino board (USB/Board communication issue).

- System failure to perform voltage measurements on the Controllino board (configuration issue).

- Process execution failure due to incorrect input or configuration (e.g., invalid pin number).

- Unfulfilled preconditions for Pullup or Logic tests